IE 410 – INTRODUCTION TO ROBOTICS

Lab-8 report

Turtlebot3 and simulation

Team M410:

201901011 – HIMANSHU DUDHATRA

201901024 – DHAVALSINH RAJ

201901100 – SHUBHAM PATEL

201901145 – GARGEY PATEL

- Installing turtlebot3

Run following commands to install turtlebot3

$ cd ~/catkin_ws/src/

$ git clone https://github.com/ROBOTIS-GIT/turtlebot3_msgs.git

$ git clone https://github.com/ROBOTIS-GIT/turtlebot3.git

$ cd ~/catkin_ws && catkin_make

After, add following line in our .bashrc file export TURTLEBOT3_MODEL=burger

Now run the following commands

$ source ~/.bashrc

$ cd ~/catkin_ws/src/

$ git clone https://github.com/ROBOTIS-GIT/turtlebot3_simulations.git

$ cd ~/catkin_ws && catkin_make

- Simulate turtlebot3 using RViz

Run the following command to simulate turtlebot3 using RViz.

$ roslaunch turtlebot3_fake turtlebot3_fake.launch

If you want to control turtlebot3 using your keyboard run the following command.

$ roslaunch turtlebot3_teleop turtlebot3_teleop_key.launch

- Simulate turtlebot3 using Gazebo

Run the following command to simulate turtlebot3 using Gazebo.

$ roslaunch turtlebot3_gazebo turtlebot3_empty_world.launch

- Managing simulation environment of turtlebot3

Let’s look at our turtlebot3 in a different environment. This environment is often used for testing SLAM and navigation algorithms. Simultaneous localization and mapping (SLAM)

concerns the problem of a robot building or updating a map of an unknown environment while simultaneously keeping track of its location in that environment.

$ roslaunch turtlebot3_gazebo turtlebot3_world.launch

Type this command and wait a few minutes for the environment to load to stimulate turtlebot3 inside a house.

$ roslaunch turtlebot3_gazebo turtlebot3_house.launch

To move the turtlebot with your keyboard, use this command in another terminal tab:

$ roslaunch turtlebot3_teleop turtlebot3_teleop_key.launch

- Simulating SLAM with TurtleBot3

Install the SLAM module in a new terminal window.

$ sudo apt install ros-melodic-slam-gmapping

$ roslaunch turtlebot3_gazebo turtlebot3_world.launch

$ roslaunch turtlebot3_slam turtlebot3_slam.launch slam_methods:=gmapping

$ roslaunch turtlebot3_gazebo turtlebot3_simulation.launch